National Textile University, Faisalabad

| Name: | Fatima Shahzad |
| --- | --- |
| Class: | BSCS(A) |
| Registration No: | 23-NTU-CS-1026 |
| Course Name: | Embedded IOT System |
| Submitted To: | Sir Nasir Mahmood |

Homework  no 1 After Mid

Question no 1

ESP32

(Web Server Webserver.cpp)

Part A short questions

Question no 1

What is the purpose of Webserver server(80); and what does port 80 represent?

Answer :

- It creates a web server on the ESP device.

- Port 80 is the default port used for web (HTTP) pages

Question no 2

Explain the role of server.on("/", handleRoot); in this program.

Answer :

- It tells the server what to do when someone opens the main webpage.

- handleRoot() runs when the home URL / is requested.

Question no 3

Why is server.handleClient(); placed inside the loop() function? What will happen if it is removed?

Answer :

- It checks again and again for new web requests from users.

- If removed, the server will not respond to any browser requests.

Question no 4

In handleRoot(), explain the statement: server.send(200, "text/html", html);

Answer :

- 200 means the request was successful.

- It sends an HTML webpage to the browser.

Question no 5

What is the difference between displaying last measured sensor values and taking a fresh DHT reading inside handleRoot()?

Answer :

- Last values show old stored sensor data.

- Fresh reading takes new data from the sensor each time.

Part B

Long Question

Describe the complete working of the ESP32 webserver-based temperature and humidity monitoring system.

Working of ESP32 Webserver-Based Temperature and Humidity Monitoring System

1. ESP32 Wi-Fi Connection and IP Address Assignment

First, the ESP32 connects to a Wi-Fi network using the SSID and password.
Once connected, the router assigns an IP address to the ESP32, which is used to open the web page in a browser.

2. Web Server Initialization and Request Handling

After Wi-Fi connection, a web server is started on port 80 using the WebServer library.
When a user opens the ESP32 IP address in a browser, the server receives the request and sends the required webpage as a response.

3. Button-Based Sensor Reading and OLED Update Mechanism

A push button is used to control sensor readings.

When the button is pressed, the ESP32 reads new temperature and humidity values from the DHT sensor and updates them on the OLED display.

4. Dynamic HTML Webpage Generation

The ESP32 creates an HTML page in the program using strings.

Sensor values are inserted into the HTML code, so the webpage shows live temperature and humidity data instead of fixed text.

5. Purpose of Meta Refresh in the Webpage

The meta refresh tag is used to automatically reload the webpage after a few seconds.
This helps display updated sensor values without manually refreshing the browser.

6. Common Issues in ESP32 Webserver Projects and Their Solutions

Wi-Fi not connecting:

Check SSID, password, and network range.

Webpage not loading:

Ensure correct IP address and port 80 is used.

Sensor gives NaN values:

Check DHT sensor wiring and add a delay between readings

OLED not updating:

Confirm correct I2C address and proper library installation.

Question no 2

Blynk Cloud

(Interfacing (blynk.cpp))

Part A short questions

Question no 1

What is the role of Blynk Template ID in an ESP32 IoT project? Why must it match the cloud template?

Answer :

- It links the ESP32 project to a specific Blynk cloud dashboard.

- It must match so the device connects to the correct cloud template.

Question no 2

Differentiate between Blynk Template ID and Blynk Auth Token.

Answer :

- Template ID identifies the project layout in Blynk Cloud.

- Auth Token uniquely identifies and authorizes the device.

Question no 3

Why does using DHT22 code with a DHT11 sensor produce incorrect readings? Mention one key difference between the two sensors.

Answer :

- Both sensors have different data formats and ranges.

- DHT11 is less accurate and works in a smaller temperature range.

Question no 4

What are Virtual Pins in Blynk? Why are they preferred over physical GPIO pins for cloud communication?

Answer :

- Virtual pins send data between ESP32 and Blynk Cloud.

- They are flexible and do not depend on physical GPIO pins.

Question no 5

What is the purpose of using BlynkTimer instead of delay() in ESP32 IoT applications?

Answer :

- BlynkTimer allows tasks to run without stopping the program.

- Delay() blocks Wi-Fi and Blynk communication.

Part B

Long Question

Explain the complete workflow of interfacing ESP32 with Blynk Cloud to display temperature and humidity values.

Answer :

Workflow of Interfacing ESP32 with Blynk Cloud to Display Temperature and Humidity

1. Creation of Blynk Template and Datastreams

First, a Blynk Template is created in the Blynk Cloud dashboard.

Inside the template, Datastreams (Virtual Pins) are added for temperature and humidity to receive sensor data from ESP32.

2. Role of Template ID, Template Name, and Auth Token

- Template ID connects the ESP32 code to the correct Blynk cloud project.

- Template Name is the project name shown in the Blynk app, while the Auth Token allows the device to securely connect to the cloud.

3. Sensor Configuration Issues (DHT11 vs DHT22)

- DHT11 and DHT22 sensors use different data formats and ranges.

- Using DHT22 code with a DHT11 sensor gives wrong values, so the correct sensor type must be selected in the code.

4. Sending Data Using Blynk.virtualWrite()

- After reading temperature and humidity from the sensor, ESP32 sends the values to Blynk Cloud using Blynk.virtualWrite()

- .These values appear instantly on the Blynk dashboard widgets linked to the same virtual pins.

5. Common Problems and Their Solutions

ESP32 not connecting to Blynk:

Check Wi-Fi credentials, Template ID, and Auth Token.

No data on Blynk app:

Ensure correct virtual pin numbers and datastream settings.

Incorrect sensor readings:

Verify sensor type, wiring, and add proper delays.

App disconnects frequently:

Avoid using delay() and use BlynkTimer instead.